Software Design Document

GPIO Driver Module

Lab 02 – Microcontrollers and Embedded Systems

| Module | GPIO Driver |
| --- | --- |
| Version | 1.1.0 |
| Author(s) | J.M. Páez Sandoval, C.A. Martínez , D. Lopez Moreno, L.A. Lugo Muñiz |
| Institution | CETYS Universidad — Facultad de Ingeniería |
| Date | 25/04/2026 |

# 1. Introduction

## 1.1  Purpose

This document describes the design and architecture of the gpio_driver module for Lab 2. It covers the updated implementation that adds full Alternate Function configuration, atomic BSRR-based pin writes, output type and speed control, and a GPIO_Status_t error-return type.

## 1.2  Scope

The gpio_driver is a low-level C module for the STM32F411RE. It uses CMSIS GPIO_TypeDef pointers and provides a clean API for configuring digital I/O, alternate functions (e.g. TIM1_CH1 on PA8), and atomic pin operations. It is used by the led, pwm, and timer modules.

## 1.3  Definitions, Acronyms, and Abbreviations

| Term | Definition |
| --- | --- |
| GPIO | General-Purpose Input/Output peripheral |
| CMSIS | Cortex Microcontroller Software Interface Standard |
| AF | Alternate Function — routes pin to a peripheral (e.g. TIM1_CH1) |
| MODER | Mode Register — 2 bits per pin: 00=Input, 01=Output, 10=AF, 11=Analog |
| OTYPER | Output Type Register — 0=Push-pull, 1=Open-drain |
| OSPEEDR | Output Speed Register — 2 bits per pin (slew rate) |
| PUPDR | Pull-Up/Pull-Down Register — 2 bits per pin |
| AFR | Alternate Function Register — AFRL (pins 0–7), AFRH (pins 8–15) |
| BSRR | Bit Set/Reset Register — atomic set (bits[15:0]) / reset (bits[31:16]) |
| IDR | Input Data Register — read-only pin states |
| ODR | Output Data Register — XOR for toggle |
| GPIO_Status_t | Return type: GPIO_OK=0, GPIO_ERROR=1 |

## 1.4  Requirements

All requirements for this module are allocated in Docs/SRS/GPIO_Driver_SRS.md (provided by the instructor without modification).

# 2. System Architecture

## 2.1  High-Level Design

The gpio_driver sits between the application/middleware layer and the hardware registers. Peripheral clock is enabled automatically inside gpio_init().

| Layer | Modules |
| --- | --- |
| Application | main.c, led.c, pwm.c — call gpio_driver APIs; no register access |
| GPIO Driver | All GPIO register operations isolated here |
| Hardware | STM32F411RE GPIO registers via CMSIS GPIO_TypeDef pointers |

## 2.2  Module Interfaces

Public API declared in gpio_driver.h. Dependencies: stm32f411xe.h, stdint.h, stddef.h. No HAL.

# 3. Detailed Design

## 3.1  Class Diagram

Figure 1. Class Diagram — gpio_driver module, enums, and dependent modules

## 3.2  Module Behavior

Figure 2. Sequence Diagram — Full pin initialization: gpio_init(), gpio_set_output_type(), gpio_set_speed(), gpio_set_af()

Figure 3. Activity Diagram — gpio_init() with NULL/range validation

Figure 4. Sequence Diagram — Atomic gpio_write() via BSRR, gpio_toggle() via ODR XOR

Figure 5. Activity Diagram — gpio_write() BSRR atomic set/reset

Figure 6. State Diagram — GPIO pin states: UNINITIALIZED → INPUT / OUTPUT / AF / ANALOG

## 3.3  GPIO Driver Module Description

gpio_init() enables the peripheral clock via the private gpio_enable_clock() helper, validates port and pin, then writes the 2-bit MODER field using a read-modify-write.

gpio_set_output_type(), gpio_set_speed(), and gpio_set_pupd() apply the same read-modify-write pattern to OTYPER (1 bit), OSPEEDR (2 bits), and PUPDR (2 bits) respectively.

gpio_set_af() selects AFRL (AFR[0]) for pins 0–7 or AFRH (AFR[1]) for pins 8–15, writing the 4-bit AF field at the correct bit offset. All functions return GPIO_ERROR if port is NULL or pin > 15.

gpio_write() uses BSRR for atomic operation: writing bit n of BSRR[15:0] sets ODR bit n HIGH; writing bit n of BSRR[31:16] resets it LOW. No read-modify-write is needed.

gpio_toggle() XORs the ODR register directly. This is non-atomic and should only be used in single-threaded contexts.

gpio_read() reads the IDR register and masks the target bit. Returns 1 if HIGH, 0 if LOW.

# 4. Application Programming Interfaces

## 4.1  Data Types and Macros

| Enum | Values | Notes |
| --- | --- | --- |
| GPIO_Mode_t | INPUT=0, OUTPUT=1, AF=2, ANALOG=3 | MODER 2-bit field value per pin |
| GPIO_OType_t | PP=0, OD=1 | OTYPER 1-bit field per pin |
| GPIO_Speed_t | LOW=0, MEDIUM=1, HIGH=2, VHIGH=3 | OSPEEDR 2-bit field per pin |
| GPIO_PUPD_t | NONE=0, PULLUP=1, PULLDOWN=2 | PUPDR 2-bit field per pin |
| GPIO_AF_t | AF0…AF15 (0–15) | AFR 4-bit field per pin |
| GPIO_Status_t | OK=0, ERROR=1 | Return type for all public functions |

## 4.2  Error Codes

| Code | Value | Definition |
| --- | --- | --- |
| GPIO_OK | 0 | Operation completed successfully |
| GPIO_ERROR | 1 | Invalid argument: port==NULL, pin>15, or AF>15 |

## 4.3  Function Definitions

### gpio_init

| Function Name | gpio_init |
| --- | --- |
| Syntax | GPIO_Status_t gpio_init(GPIO_TypeDef *port, uint8_t pin, GPIO_Mode_t mode) |
| Parameters (in) | port — GPIOA…GPIOE pointerpin — 0–15mode — INPUT, OUTPUT, AF, or ANALOG |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | GPIO_OK or GPIO_ERROR |
| Description | Enables GPIO peripheral clock, validates arguments, then writes MODER[2n+1:2n] = mode. Must be called before any other function on the pin. |
| Constraints | Does not configure OTYPER, OSPEEDR, PUPDR, or AFR — call gpio_set_* functions separately after gpio_init(). |
| Available via | gpio_driver.h |

### gpio_set_output_type

| Function Name | gpio_set_output_type |
| --- | --- |
| Syntax | GPIO_Status_t gpio_set_output_type(GPIO_TypeDef *port, uint8_t pin, GPIO_OType_t otype) |
| Parameters (in) | port, pin, otype — PP or OD |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | GPIO_OK or GPIO_ERROR |
| Description | Configures OTYPER[n] = otype. Push-pull can source and sink current; open-drain only sinks. |
| Constraints | Only meaningful for output-mode pins. |
| Available via | gpio_driver.h |

### gpio_set_speed

| Function Name | gpio_set_speed |
| --- | --- |
| Syntax | GPIO_Status_t gpio_set_speed(GPIO_TypeDef *port, uint8_t pin, GPIO_Speed_t speed) |
| Parameters (in) | port, pin, speed — LOW/MEDIUM/HIGH/VHIGH |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | GPIO_OK or GPIO_ERROR |
| Description | Sets OSPEEDR[2n+1:2n] = speed. Controls output slew rate. Use GPIO_SPEED_HIGH for PWM output pins. |
| Constraints | Higher speed increases EMI; use the lowest speed sufficient for the application. |
| Available via | gpio_driver.h |

### gpio_set_pupd

| Function Name | gpio_set_pupd |
| --- | --- |
| Syntax | GPIO_Status_t gpio_set_pupd(GPIO_TypeDef *port, uint8_t pin, GPIO_PUPD_t pupd) |
| Parameters (in) | port, pin, pupd — NONE/PULLUP/PULLDOWN |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | GPIO_OK or GPIO_ERROR |
| Description | Configures PUPDR[2n+1:2n]. Enables or disables internal pull resistors (~40 kΩ typical). |
| Constraints | External resistors take precedence over internal ones. |
| Available via | gpio_driver.h |

### gpio_set_af

| Function Name | gpio_set_af |
| --- | --- |
| Syntax | GPIO_Status_t gpio_set_af(GPIO_TypeDef *port, uint8_t pin, GPIO_AF_t af) |
| Parameters (in) | port, pin (0–15), af (AF0–AF15) |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | GPIO_OK or GPIO_ERROR (if af > 15) |
| Description | Writes the 4-bit AF field in AFR[0] (pins 0–7) or AFR[1] (pins 8–15). For PA8: gpio_set_af(GPIOA, 8, GPIO_AF1) maps to TIM1_CH1. |
| Constraints | Pin must be in GPIO_MODE_AF via gpio_init() first. Refer to STM32F411 datasheet Table 9 for AF assignments. |
| Available via | gpio_driver.h |

### gpio_write

| Function Name | gpio_write |
| --- | --- |
| Syntax | void gpio_write(GPIO_TypeDef *port, uint8_t pin, uint8_t value) |
| Parameters (in) | port, pin, value — 1=HIGH, 0=LOW |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | void |
| Description | Drives output pin atomically via BSRR. value!=0 → BSRR[pin]=1 (set); value==0 → BSRR[pin+16]=1 (reset). No read-modify-write needed. |
| Constraints | Pin must be configured as OUTPUT. No NULL/range check (void function). |
| Available via | gpio_driver.h |

### gpio_toggle

| Function Name | gpio_toggle |
| --- | --- |
| Syntax | void gpio_toggle(GPIO_TypeDef *port, uint8_t pin) |
| Parameters (in) | port, pin |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | void |
| Description | Inverts current pin output state via XOR on ODR. Used by led_toggle() in the LED blink loop. |
| Constraints | Non-atomic (read-modify-write on ODR). Do not use in concurrent contexts. |
| Available via | gpio_driver.h |

### gpio_read

| Function Name | gpio_read |
| --- | --- |
| Syntax | uint8_t gpio_read(GPIO_TypeDef *port, uint8_t pin) |
| Parameters (in) | port, pin |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | 1 if HIGH, 0 if LOW |
| Description | Reads current pin state from IDR: (IDR >> pin) & 0x1U. |
| Constraints | Reads IDR regardless of pin mode. |
| Available via | gpio_driver.h |

## 4.4  Internal Function Definitions

### gpio_enable_clock  (static)

| Function Name | gpio_enable_clock |
| --- | --- |
| Syntax | static void gpio_enable_clock(GPIO_TypeDef *port) |
| Parameters (in) | port — GPIOA…GPIOE pointer |
| Return value | void |
| Description | Sets the corresponding GPIOxEN bit in RCC->AHB1ENR. Called automatically at the start of gpio_init(). Not accessible outside gpio_driver.c. |
| Constraints | Only handles GPIOA–GPIOE. Extend for GPIOH if needed. |

# 5. Future Enhancements

- Add GPIOH support in gpio_enable_clock() for NUCLEO boards that use it.

- Implement gpio_lock() using the LCKR register to protect critical pin configurations.

- Add gpio_set_exti() to configure EXTI interrupt lines for edge-triggered input events.

- Add input validation to gpio_write(), gpio_toggle(), and gpio_read() (currently void functions with no check).