Software Design Document (SDD)

ADC Driver — STM32F411RE

| Project | Lab Assignment 03 — Microcontrollers and Embedded Systems |
| --- | --- |
| Author | Luis Angel Lugo Muñiz (37915, B6) |
| Course | MSE — Sixth Semester, CETYS Universidad Tijuana |
| Module | adc_driver |
| Target | STM32F411RE on NUCLEO-F411RE |
| Toolchain | arm-none-eabi-gcc + CMSIS, bare-metal |
| Date | May 06, 2026 |

# 1. Introduction

## 1.1 Purpose

This document describes the design and architecture of the ADC driver module for the STM32F411RE microcontroller, based on the requirements defined in the SRS-ADC_Driver. It provides details on data structures, register-level configuration, control flow, and the public API consumed by the higher-level sensor module. The intended audience is firmware developers integrating or maintaining this driver and reviewers verifying SRS compliance.

## 1.2 Scope

The ADC driver is a low-level software component that abstracts the STM32F4 ADC peripheral registers (CR1, CR2, SQRx, SMPRx, SR, DR) and exposes a clean, status-returning C API. The application uses this driver indirectly through the sensor module to convert the analog voltage at PA1 (ADC1_IN1, the wiper of a potentiometer) into a 12-bit digital value that drives the duty cycle of a PWM output on PA8.

The driver supports single-channel, single-conversion, polling-mode operation on ADC1. Continuous mode, scan mode, DMA, and injected channels are out of scope for this lab; their hooks are documented as future enhancements (Section 5).

## 1.3 Definitions, Acronyms, and Abbreviations

| Term | Definition |
| --- | --- |
| ADC | Analog-to-Digital Converter peripheral |
| ADON | ADC ON bit (CR2 bit 0) — powers the peripheral |
| Channel | Input multiplexer line connected to the ADC (channels 0–18 on F411RE) |
| CMSIS | Cortex Microcontroller Software Interface Standard (ARM) |
| DR | Data Register — holds the most recent conversion result |
| EOC | End Of Conversion flag (SR bit 1) |
| OVR | Overrun flag (SR bit 5) — set when DR is overwritten before being read |
| Resolution | Number of bits in the digital result (6, 8, 10, or 12) |
| SQR | Sequence Register — defines which channels are converted and in what order |
| SMPR | Sample-time Register — programs the per-channel sampling time in cycles |
| SWSTART | Software Start bit (CR2 bit 30) — triggers a regular conversion |
| Vref | Reference voltage; on NUCLEO-F411RE Vref+ = 3.3 V |

## 1.4 Requirements

All functional requirements specific to this module are allocated in SRS-ADC_Driver.md. Section 4 of this SDD includes a traceability table mapping every implemented function to its corresponding SRS requirement (FR-1 through FR-9).

# 2. System Architecture

## 2.1 High-Level Design

The system follows a strict three-layer architecture. Each layer depends only on the layer immediately below it, which makes the driver portable and the application testable in isolation.

┌──────────────────────────────────────────────────────────────┐

│                      Application Layer                       │

│                          (main.c)                            │

│   super-loop: read sensor → map to duty → update PWM → blink │

└──────────────────────────────────────────────────────────────┘

│

▼

┌──────────────────────────────────────────────────────────────┐

│                 Hardware Abstraction Layer                   │

│   sensor (PA1)     │     pwm (PA8)     │     led (PA5)       │

└──────────────────────────────────────────────────────────────┘

│

▼

┌──────────────────────────────────────────────────────────────┐

│                       Driver Layer                           │

│   adc_driver  │  tim_driver  │  gpio_driver  │  timer (TIM2) │

└──────────────────────────────────────────────────────────────┘

│

▼

┌──────────────────────────────────────────────────────────────┐

│                       Hardware Layer                         │

│      STM32F411RE peripheral registers (via CMSIS headers)    │

└──────────────────────────────────────────────────────────────┘

The ADC driver sits in the Driver Layer. It does not call any other driver — it only writes to the ADC1 register block exposed through CMSIS.

## 2.2 Module Interfaces

Public API: All functions defined in adc_driver.h. The seven public functions are: adc_init, adc_set_channel, adc_enable, adc_disable, adc_start, adc_poll_eoc, adc_read.

Dependencies:

• stm32f411xe.h (CMSIS device header) — provides ADC_TypeDef, ADC1, and bit-mask macros (ADC_CR1_RES, ADC_CR2_ADON, ADC_SR_EOC, etc.).

• stdint.h, stddef.h — fixed-width integer types and NULL.

Consumers:

• sensor.c — wraps adc_driver to provide a high-level read of the potentiometer voltage. The application never calls adc_driver functions directly.

Pre-conditions enforced outside the driver:

• The ADC1 peripheral clock must be enabled in RCC->APB2ENR before adc_init() is invoked. This is the responsibility of the sensor module (which also configures PA1 as analog through gpio_driver).

• Vref+ must be supplied externally (provided by the NUCLEO-F411RE board at 3.3 V).

# 3. Detailed Design

## 3.1 Class Diagram

The driver is procedural C code, but the relationships between data types and functions can be expressed as a UML-style component view:

┌─────────────────────────────────────────┐

│          «module» adc_driver            │

├─────────────────────────────────────────┤

│ + adc_init(adc, config)    : Status     │

│ + adc_set_channel(...)     : Status     │

│ + adc_enable(adc)          : Status     │

│ + adc_disable(adc)         : Status     │

│ + adc_start(adc)           : Status     │

│ + adc_poll_eoc(adc, t/o)   : Status     │

│ + adc_read(adc, *result)   : Status     │

│ - set_sample_time(...)                  │

└─────────────────────────────────────────┘

│ uses             │ produces

▼                  ▼

┌──────────────────┐    ┌─────────────────┐

│  ADC_Config_t    │    │  ADC_Status_t   │

├──────────────────┤    ├─────────────────┤

│ resolution       │    │ ADC_OK          │

│ align            │    │ ADC_ERROR_NULL  │

│ channel          │    │ ADC_ERROR_CHAN  │

│ sampleTime       │    │ ADC_ERROR_TMO   │

└──────────────────┘    │ ADC_ERROR_OVR   │

└─────────────────┘

Figure 1. Component view of the ADC driver. Public functions take a pointer to ADC_TypeDef (CMSIS) and either a configuration struct or simple scalars, and always return an ADC_Status_t code.

## 3.2 Module Behavior

### 3.2.1 Driver Initialization (Sequence)

The standard call sequence performed by the sensor module on system startup:

sensor                  adc_driver                 ADC1 registers

│                          │                            │

│  adc_init(ADC1, &cfg) ──▶│                            │

│                          │  CR1 ← RES bits            │

│                          │  CR2 ← EOCS, ALIGN clear   │

│                          │  SQR1 ← L = 0              │

│                          │  SQR3 ← channel 1          │

│                          │  SMPR2 ← 84 cycles for ch1 │

│  ◀───── ADC_OK ──────────│                            │

│                          │                            │

│  adc_enable(ADC1) ──────▶│                            │

│                          │  CR2.ADON ← 1              │

│                          │  busy-wait ~3 µs           │

│  ◀───── ADC_OK ──────────│                            │

Figure 2. Sequence — driver initialization. Note that adc_init() does not power on the peripheral; this is a deliberate split (see Section 3.3) so the caller can configure all channels before any conversion is possible.

### 3.2.2 Single Conversion (Activity)

┌────────────────────────────┐

│   adc_start(ADC1)          │

│   (sets CR2.SWSTART = 1)   │

└────────────┬───────────────┘

│

▼

┌────────────────────────────┐

│ adc_poll_eoc(ADC1, t/o)    │

└────────────┬───────────────┘

│

┌────────┴─────────┐

▼                  ▼

┌──────────────┐   ┌──────────────┐

│  SR.OVR = 1? │──▶│ clear OVR    │

└──────┬───────┘   │ return ERR   │

│ no        └──────────────┘

▼

┌──────────────┐

│  SR.EOC = 1? │── yes ──▶ return OK

└──────┬───────┘

│ no

▼

┌──────────────┐

│ timeout--    │── 0 ──▶ return TIMEOUT

└──────┬───────┘

│ > 0

└──────► loop

Figure 3. Activity — polled conversion with overrun and timeout guards.

### 3.2.3 Driver State Diagram

┌─────────────┐

│   RESET     │  (registers in default state)

└──────┬──────┘

│ adc_init()

▼

┌─────────────┐

│ CONFIGURED  │  (channel + sample time set)

└──────┬──────┘

│ adc_enable()

▼

┌─────────────┐  adc_start()    ┌──────────────┐

│   IDLE      │ ───────────────▶│ CONVERTING   │

│ (ADON = 1)  │                 │ (EOC clears  │

│             │ ◀───────────────│  on DR read) │

└──────┬──────┘   adc_read()    └──────────────┘

│ adc_disable()

▼

┌─────────────┐

│ CONFIGURED  │  (back to step 2)

└─────────────┘

Figure 4. State diagram of the ADC peripheral as managed by this driver.

## 3.3 ADC Driver — Design Rationale

Why a configuration struct (ADC_Config_t)?

The four parameters that must be set together (resolution, alignment, channel, sample time) are passed as a struct so adding a new field later (e.g., continuousMode) does not break existing callers. The struct is taken const * so the caller can declare it static const in flash and avoid stack copies.

Why split adc_init() from adc_enable()?

The reference manual (RM0383, §11.3.1) requires ADON = 0 while CR1/CR2/SQRx are written. Splitting the API forces the caller into the correct sequence: configure first, power on second. It also lets the application disable the ADC for low-power modes without re-configuring on wake-up.

Why polling instead of EOC interrupt?

A 12-bit conversion with a 84-cycle sample time at the default 16 MHz ADCCLK takes roughly 6 µs. The sensor_read call site is the slowest stage of the super-loop already, and the simplicity of polling makes the driver easier to reason about for the lab's modular-design rubric. Interrupt-driven conversion is listed as a future enhancement.

Why explicit overrun handling?

The STM32F4 ADC is a fast peripheral and OVR can fire if a higher-priority interrupt delays the DR read across two conversions. Returning ADC_ERROR_OVERRUN instead of silently giving stale data lets the upper layer log the event or retry.

Why a timeout in adc_poll_eoc()?

A driver that can busy-wait forever is a single point of failure for the entire firmware. The timeout (ADC_TIMEOUT_CYCLES = 100,000) is generous enough (~6 ms at 16 MHz) that legitimate conversions never trip it, but short enough that a wedged ADC will surface as a fault inside one super-loop iteration instead of locking the system.

Sample time choice (84 cycles).

The sensor module selects ADC_SMP_84_CYCLES because the potentiometer wiper has non-trivial source impedance (≈ 5 kΩ at midpoint). Per the F411RE datasheet (DS9716, Table 67), 84 cycles is the recommended minimum for sources up to ~10 kΩ at 12-bit resolution.

# 4. Application Programming Interfaces

## 4.1 Data Types and Macros

### ADC_Resolution_t (enum)

Selects the ADC conversion resolution by writing CR1.RES[25:24].

| Member | Value | Range |
| --- | --- | --- |
| ADC_RES_12BIT | 0x00 | 0–4095 |
| ADC_RES_10BIT | 0x01 | 0–1023 |
| ADC_RES_8BIT | 0x02 | 0–255 |
| ADC_RES_6BIT | 0x03 | 0–63 |

### ADC_Align_t (enum)

Selects right- or left-alignment in the data register (CR2.ALIGN[11]).

| Member | Value | Notes |
| --- | --- | --- |
| ADC_ALIGN_RIGHT | 0x00 | Default; result occupies LSBs of DR |
| ADC_ALIGN_LEFT | 0x01 | Result occupies MSBs of DR (for 12-bit fixed-point math) |

### ADC_SampleTime_t (enum)

Per-channel sample time programmed in SMPRx (3-bit field, eight values).

| Member | Cycles | Member | Cycles |
| --- | --- | --- | --- |
| ADC_SMP_3_CYCLES | 3 | ADC_SMP_84_CYCLES | 84 |
| ADC_SMP_15_CYCLES | 15 | ADC_SMP_112_CYCLES | 112 |
| ADC_SMP_28_CYCLES | 28 | ADC_SMP_144_CYCLES | 144 |
| ADC_SMP_56_CYCLES | 56 | ADC_SMP_480_CYCLES | 480 |

### ADC_Config_t (struct)

Aggregates all parameters passed to adc_init().

| Member | Type | Notes |
| --- | --- | --- |
| resolution | ADC_Resolution_t | Conversion resolution |
| align | ADC_Align_t | Data alignment in DR |
| channel | uint8_t | ADC input channel, 0–18 |
| sampleTime | ADC_SampleTime_t | Sample time for the configured channel |

### Macros

| Macro | Value | Purpose |
| --- | --- | --- |
| ADC_STAB_DELAY_CYCLES | 48U | NOPs after ADON to ensure stabilization |
| ADC_TIMEOUT_CYCLES | 100000U | Default poll budget for adc_poll_eoc() |
| ADC_MAX_CHANNEL | 18U | Highest valid channel on STM32F411RE |

## 4.2 Error Codes

All public functions return ADC_Status_t.

| Code | Value | Notes |
| --- | --- | --- |
| ADC_OK | 0 | Operation completed successfully |
| ADC_ERROR_NULL | 1 | NULL pointer passed as argument (NFR-2) |
| ADC_ERROR_CHANNEL | 2 | Channel number > 18 (NFR-2) |
| ADC_ERROR_TIMEOUT | 3 | EOC did not assert within the timeout budget |
| ADC_ERROR_OVERRUN | 4 | SR.OVR was set; data integrity lost |

## 4.3 Function Definitions

### adc_init

| Function name | adc_init |
| --- | --- |
| Syntax | ADC_Status_t adc_init(ADC_TypeDef *adc, const ADC_Config_t *config) |
| Parameters (in) | adc — ADC peripheral pointer (e.g., ADC1). config — pointer to a populated configuration struct. |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | ADC_Status_t — ADC_OK, ADC_ERROR_NULL, or ADC_ERROR_CHANNEL. |
| Description | Configures CR1 (resolution), CR2 (alignment, single conversion, EOCS=1), SQR1 (sequence length = 1), and delegates channel + sample time to adc_set_channel. Does not power on the ADC. |
| Constraints | The ADC peripheral clock must already be enabled (RCC->APB2ENR). Function does not validate the resolution or alignment fields beyond the type system. |
| Available via | adc_driver.h |
| SRS trace | FR-1 (init), FR-2 (peripheral enable preparation) |

### adc_set_channel

| Function name | adc_set_channel |
| --- | --- |
| Syntax | ADC_Status_t adc_set_channel(ADC_TypeDef *adc, uint8_t channel, ADC_SampleTime_t smp) |
| Parameters (in) | adc — ADC peripheral pointer. channel — 0 to 18. smp — sample time selection. |
| Return value | ADC_OK, ADC_ERROR_NULL, or ADC_ERROR_CHANNEL. |
| Description | Writes the channel number into SQR3.SQ1[4:0] and the 3-bit sample time into the corresponding field of SMPR2 (channels 0–9) or SMPR1 (channels 10–18). |
| Constraints | Only configures the first conversion slot; multi-channel sequences are not supported. |
| Available via | adc_driver.h |
| SRS trace | FR-3 (channel selection) |

### adc_enable

| Function name | adc_enable |
| --- | --- |
| Syntax | ADC_Status_t adc_enable(ADC_TypeDef *adc) |
| Parameters (in) | adc — ADC peripheral pointer. |
| Return value | ADC_OK or ADC_ERROR_NULL. |
| Description | Sets CR2.ADON to power on the peripheral, then burns ADC_STAB_DELAY_CYCLES NOPs to satisfy the minimum stabilization time before any conversion can be triggered. |
| Constraints | Busy-waits with NOPs (no SysTick dependency). Delay is calibrated for ~16 MHz core; faster cores will over-wait but remain correct. |
| Available via | adc_driver.h |
| SRS trace | FR-4 (enable peripheral) |

### adc_disable

| Function name | adc_disable |
| --- | --- |
| Syntax | ADC_Status_t adc_disable(ADC_TypeDef *adc) |
| Parameters (in) | adc — ADC peripheral pointer. |
| Return value | ADC_OK or ADC_ERROR_NULL. |
| Description | Clears CR2.ADON; further adc_start calls have no effect until adc_enable is invoked again. |
| Constraints | Does not flush in-flight conversions; the caller must ensure no adc_start is pending. |
| Available via | adc_driver.h |
| SRS trace | FR-5 (disable peripheral; covers the inverse of FR-4) |

### adc_start

| Function name | adc_start |
| --- | --- |
| Syntax | ADC_Status_t adc_start(ADC_TypeDef *adc) |
| Parameters (in) | adc — ADC peripheral pointer. |
| Return value | ADC_OK or ADC_ERROR_NULL. |
| Description | Sets CR2.SWSTART, triggering a single regular conversion on the channel previously configured. Hardware clears SWSTART automatically when the conversion begins. |
| Constraints | Requires adc_enable to have been called and the stabilization delay to have elapsed. |
| Available via | adc_driver.h |
| SRS trace | FR-6 (start single conversion via software trigger) |

### adc_poll_eoc

| Function name | adc_poll_eoc |
| --- | --- |
| Syntax | ADC_Status_t adc_poll_eoc(ADC_TypeDef *adc, uint32_t timeout) |
| Parameters (in) | adc — ADC peripheral pointer. timeout — maximum loop iterations to wait. |
| Return value | ADC_OK, ADC_ERROR_NULL, ADC_ERROR_TIMEOUT, or ADC_ERROR_OVERRUN. |
| Description | Busy-loops on SR until either EOC asserts (success), OVR asserts (data lost — flag is cleared and error returned), or the timeout counter reaches zero. |
| Constraints | The timeout unit is loop iterations, not microseconds — actual elapsed time depends on core clock and compiler optimizations. The conservative default macro ADC_TIMEOUT_CYCLES corresponds to ~6 ms at 16 MHz with -O0. |
| Available via | adc_driver.h |
| SRS trace | FR-7 (wait for EOC); NFR-2 (error handling) |

### adc_read

| Function name | adc_read |
| --- | --- |
| Syntax | ADC_Status_t adc_read(ADC_TypeDef *adc, uint16_t *result) |
| Parameters (in) | adc — ADC peripheral pointer. |
| Parameters (out) | result — pointer to a 16-bit variable receiving the raw conversion value. |
| Return value | ADC_OK or ADC_ERROR_NULL. |
| Description | Reads DR (which automatically clears SR.EOC in hardware) and stores the lower 16 bits in *result. Caller is responsible for interpreting the alignment and bit width. |
| Constraints | Does not check that EOC was actually set — the caller must invoke adc_poll_eoc first or accept that *result may contain a stale value. |
| Available via | adc_driver.h |
| SRS trace | FR-8 (read regular conversion result) |

## 4.4 Internal Function Definitions

### set_sample_time (static helper)

| Function name | set_sample_time |
| --- | --- |
| Syntax | static void set_sample_time(ADC_TypeDef *adc, uint8_t channel, ADC_SampleTime_t smp) |
| Parameters (in) | adc — ADC peripheral pointer. channel — 0–18. smp — sample time selection. |
| Return value | void |
| Description | Writes the 3-bit sample-time field into SMPR1 or SMPR2 depending on the channel. For channels 0–9 the bit-field offset is channel * 3 in SMPR2; for channels 10–18 the offset is (channel - 10) * 3 in SMPR1. The existing field is cleared first to allow re-configuration. |
| Constraints | Internal helper — does not validate inputs (callers adc_init and adc_set_channel already do). Not exported in the header. |

## Traceability Summary

| SRS Requirement | Description | Implementation |
| --- | --- | --- |
| FR-1 | Initialize the ADC subsystem | adc_init |
| FR-2 | Activate the ADC peripheral | adc_enable |
| FR-3 | Select valid regular channels | adc_set_channel |
| FR-4 | Configure injected channels | Not implemented (out of scope, see §5) |
| FR-5 | Start single conversion | adc_start (preceded by adc_init/adc_enable) |
| FR-6 | Start continuous conversion | Not implemented (out of scope, see §5) |
| FR-7 | Start injected conversion | Not implemented (out of scope, see §5) |
| FR-8 | Read regular conversion result | adc_read (with adc_poll_eoc for synchronization) |
| FR-9 | Read injected channel result | Not implemented (out of scope, see §5) |
| NFR-1 | Lightweight and efficient | Polling, no allocations, ~250 bytes of .text |
| NFR-2 | Error handling for invalid inputs | ADC_Status_t returned by every function |
| NFR-3 | Real-time performance | Bounded adc_poll_eoc timeout; total conversion ≤ 6 µs |

# 5. Future Enhancements

The current implementation covers the lab requirements (regular conversion on a single channel, polling mode). The following extensions are reasonable next steps and are explicitly listed because the SRS scopes them as future work:

- Injected channel support (FR-4, FR-7, FR-9). Add adc_set_injected_channel, adc_start_injected, and adc_read_injected_data wrapping JSQRx, JSWSTART, and JDRx. Useful for sampling synchronized to a timer event (e.g., motor-control current sensing).

- Continuous mode (FR-6). Set CR2.CONT and replace the EOC poll with a circular-buffer DMA stream, so the application can read the latest sample in O(1) without triggering each conversion.

- DMA support. Configure DMA2 stream 0/4 for ADC1 to remove the CPU from the data path entirely. This is the natural extension when the application grows to multiple channels.

- Multi-channel scan. Support sequences of length > 1 by accepting an ADC_Channel_t[] and writing SQR1/SQR2/SQR3 with the full sequence and length.

- Calibration helper. STM32F4 ADCs do not require user calibration, but future ports to families that do (G0, L4+) would benefit from an adc_calibrate function.

- Interrupt-driven conversion. Enable EOCIE in CR1 and provide a registered callback so the application can sleep between conversions in a low-power mode.

- Resolution-aware read. adc_read could shift the result based on the configured alignment/resolution so the caller never sees raw register layout. Currently this is left to the consumer (sensor.c) for transparency.